# Theory — Two-Camera Projective Geometry

Deriving the relationship between the image coordinates of a point P(X, Y, Z) as seen by a static camera (camera 1) and a second camera (camera 2) at an oblique orientation.

## 1. Problem statement

Let a 3-D point P = (X, Y, Z)^T lie within the field of view of two cameras:

• Camera 1 is *static* (its coordinate frame is taken as the world frame).

• Camera 2 is displaced from camera 1 by a rotation R and a translation t (an *oblique* orientation).

We wish to derive the mathematical relationship between P's image coordinates in the two cameras,

p_1 = (x_1, y_1, 1)^T and p_2 = (x_2, y_2, 1)^T (homogeneous pixel coordinates).

The answer is the epipolar constraint, which ultimately takes the compact form

p_2^T F p_1 = 0

where F is the *fundamental matrix* (or, for calibrated cameras, the *essential matrix* E). This section derives it from first principles.

## 2. Assumptions and justification

| # | Assumption | Justification |
| --- | --- | --- |
| A1 | Both cameras obey the pinhole model (no lens distortion). | The standard imaging model; distortion is removed in Step 1 by cv2.calibrateCamera, so the images are effectively pinhole projections. |
| A2 | The two cameras are related by a rigid-body transformation (R, t). | Both cameras are static and mounted rigidly; a rigid motion (rotation + translation) is the most general mapping between two fixed Euclidean frames. |
| A3 | The point P is in front of both cameras and at finite depth (Z_1 > 0, Z_2 > 0). | P is within the FoV of both cameras (given), so its depth along each optical axis is positive and finite. |
| A4 | The scene is static — P does not move between the two views. | The cameras are simultaneous/static; otherwise the correspondence would not be a single 3-D point. |
| A5 | The camera intrinsics K_1, K_2 are known (for the essential-matrix form). | Intrinsics are obtained from Step 1 calibration. If intrinsics are unknown, the weaker fundamental matrix still holds (projective, not metric). |
| A6 | R is a proper rotation: R^TR = I, det(R) = +1. | Rotations of physical rigid bodies lie in SO(3). |

## 3. Static parameters and variables

| Symbol | Meaning | How determined |
| --- | --- | --- |
| K_1, K_2 (3x3) | Intrinsic matrices: [[fx, 0, cx],[0, fy, cy],[0, 0, 1]] | Camera calibration (Step 1). fx, fy = focal lengths in pixels; cx, cy = principal point. |
| R (3x3) | Rotation from camera-1 frame to camera-2 frame | Estimated from correspondences (see Sec. 7), or from a calibration rig. |
| t (3x1) | Translation from camera-1 frame to camera-2 frame (position of camera-1's center in camera-2's frame) | Same as R; known up to scale for E. |
| E = [t]x R (3x3) | Essential matrix | Product of t and R; recovered from E by SVD. |
| F = K_2^-T E K_1^-1 (3x3) | Fundamental matrix | From E and intrinsics, or directly from >= 8 correspondences (8-point algorithm). |
| b = ¦t¦ | Baseline (distance between the two optical centers) | Norm of t; provides the absolute scale (ambiguous in E/F). |
| e_1, e_2 | Epipoles (projection of one camera's center into the other's image) | e_1 = K_1(-R^Tt), e_2 = K_2 t (up to scale). |
| X, X_2 | P in camera-1 / camera-2 coordinates | Unknown (the 3-D point being observed). |
| x_1, x_2 | Normalized image coords (= X/Z, = X_2/Z_2) | From pixels via x = K^-1 p. |

Here [t]x denotes the skew-symmetric (cross-product) matrix of t,

[  0   -t_3   t_2 ]

[t]x =  [  t_3   0   -t_1 ]

[ -t_2   t_1   0  ]

such that [t]x v = t x v for any vector v.

## 4. Single-camera pinhole model (recap)

For a camera with intrinsic matrix K, a point X = (X, Y, Z)^T projects to pixel p by

Z * p~ = K * X          =>          x = fx*(X/Z) + cx,   y = fy*(Y/Z) + cy

where p~ = (x, y, 1)^T is the homogeneous pixel vector. Dividing out Z, the normalized image point is

x_hat = X / Z = (X/Z, Y/Z, 1)^T

so that p = K x_hat. This is the same perspective-projection equation used for the single-camera measurement in Step 2.

## 5. Two-camera setup (relative pose)

Let camera 1's frame be the world frame, so for P = X:

X_1 = X                    (camera-1 coordinates)

X_2 = R X_1 + t             (camera-2 coordinates)

Using the normalized projections,

X_1 = Z_1 x_1        where  x_1 = X_1/Z_1

X_2 = Z_2 x_2        where  x_2 = X_2/Z_2

Substituting into the relative-pose equation:

Z_2 x_2 = Z_1 R x_1 + t                                  (1)

## 6. Derivation of the epipolar constraint

Starting from Eq. (1), premultiply both sides by the skew-symmetric matrix [t]x:

[t]x (Z_2 x_2) = Z_1 [t]x R x_1 + [t]x t

Since [t]x t = t x t = 0, this simplifies to

[t]x (Z_2 x_2) = Z_1 [t]x R x_1                           (2)

Now take the inner product of both sides with x_2 (left-multiply by x_2^T):

x_2^T [t]x (Z_2 x_2) = Z_1 x_2^T [t]x R x_1

The left-hand side is Z_2 * x_2^T (t x x_2). Because the cross product t x x_2 is perpendicular to x_2, the dot product vanishes:

x_2^T (t x x_2) = 0   =>   left-hand side = 0

Therefore

x_2^T [t]x R x_1 = 0   (normalized coordinates)

Defining the essential matrix

E = [t]x R

gives the compact form

x_2^T E x_1 = 0   (epipolar constraint, normalized coordinates)

## 7. From normalized to pixel coordinates — the fundamental matrix

Using x_1 = K_1^-1 p_1 and x_2 = K_2^-1 p_2:

(K_2^-1 p_2)^T E (K_1^-1 p_1) = 0

p_2^T (K_2^-T E K_1^-1) p_1 = 0

Defining the fundamental matrix

F = K_2^-T E K_1^-1 = K_2^-T [t]x R K_1^-1

yields the pixel-domain relationship

p_2^T F p_1 = 0

For the special (common) case of identical cameras K_1 = K_2 = K, this is simply

F = K^-T E K^-1.

## 8. Geometric interpretation

• Epipolar line. For a point p_1 in image 1, the possible locations of the corresponding point in image 2 are constrained to the line l_2 = F p_1 (in homogeneous coordinates). The correspondence p_2 must satisfy p_2^T l_2 = 0, i.e. p_2 lies on l_2. This is the one-dimensional constraint that turns 2-D point matching into a line search.

• Epipoles. The epipole e_2 is the projection of camera 1's optical center into image 2, and e_1 is the projection of camera 2's center into image 1. They are the (right and left) null vectors of F:

F e_1 = 0,     F^T e_2 = 0

with e_1 = K_1(-R^T t) and e_2 = K_2 t. All epipolar lines in image 2 pass through e_2.

• Degrees of freedom.

- E has 5 dof: 3 for R and 2 for the direction of t (t is known only up to scale).

- F has 7 dof: a 3x3 matrix of rank 2, defined up to scale (9 - 1 scale - 1 rank constraint).

## 9. Determining the parameters

Intrinsics (K_1, K_2). Obtained once by calibration (Step 1): the camera matrix gives fx, fy, cx, cy, and the distortion coefficients correct the images.

Fundamental matrix F (from image correspondences only). Given N >= 8 point correspondences (p_1_i <-> p_2_i), each pair contributes one linear equation in the 9 entries of F (writing p_2^T F p_1 = 0 as a dot product (p_2 x p_1)^T vec(F) = 0). Stack N equations and solve the resulting linear system (the 8-point algorithm); enforce rank(F) = 2 by setting the smallest singular value to zero. (Normalize point coordinates first for numerical stability.)

Essential matrix E and the relative pose (R, t). Once F is known and the cameras are calibrated,

E = K_2^T F K_1

Decompose E by SVD: E = U diag(s, s, 0) V^T (enforce the two equal singular values). The rotation and translation are recovered from U and V:

R = U W V^T   (or U W^T V^T),      t = +/- u_3

where   W = [[0, -1, 0],[1, 0, 0],[0, 0, 1]]   and u_3 = third column of U

This yields four candidate (R, t) pairs; the correct one is selected by the *cheirality* test — triangulating P and keeping the solution for which P lies in front of both cameras (Z_1 > 0 and Z_2 > 0).

Absolute scale (baseline). E and F determine R and the direction of t, but not |t| — the geometry is defined up to an overall scale. The baseline b = |t| must be fixed by a known physical measurement (e.g., the measured object distance in Step 3, or a calibration object of known size).

## 10. Connection to the single-camera measurement (Step 2)

The Step-2 measurement L = (dx / fx) * Z is the fronto-parallel special case of this geometry. If camera 2 differs from camera 1 by a pure horizontal translation (R = I, t = (b, 0, 0)^T — a *rectified stereo* pair), the epipolar constraint reduces to horizontal epipolar lines, and the horizontal disparity d = x_1 - x_2 of a point P gives its depth directly:

Z = fx * b / d ,   X = (x_1 - cx) * Z / fx ,   Y = (y_1 - cy) * Z / fy

Thus the two-camera relationship derived here is the general setting of which the single-camera perspective-projection measurement is a limiting case.